Chapter A3

# AXI Transport

This chapter describes the channel transport used in AXI. It contains the following sections:

- A3.1 Clock and reset

- A3.2 Channel handshake

- A3.3 Write and read channels

- A3.4 Relationships between the channels

- A3.5 Dependencies between channel handshake signals

- A3.6 Snoop channels

A3.1. Clock and reset

# A3.1 Clock and reset

This section describes the requirements for implementing the AXI global clock and reset signals ACLK and ARESETn.

| A3.1.1 | Clock Each AXI interface has a single clock signal, ACLK. All input signals are sampled on the rising edge of ACLK. All output signal changes can only occur after the rising edge of ACLK. On Manager and Subordinate interfaces, there must be no combinatorial paths between input and output signals. |
| --- | --- |
| A3.1.2 | Reset |

The AXI protocol uses a single active-LOW reset signal, ARESETn. The reset signal can be asserted asynchronously, but deassertion can only be synchronous with a rising edge of ACLK.

During reset the following interface requirements apply:

- A Manager interface must drive AWVALID, WVALID, and ARVALID LOW.

- A Subordinate interface must drive BVALID and RVALID LOW.

- All other signals can be driven to any value.

The earliest point after reset that a Manager is permitted to begin driving AWVALID, WVALID, or ARVALID HIGH is at a rising ACLK edge after ARESETn is HIGH. Figure A3.1 shows the earliest point b after reset that AWVALID, WVALID, or ARVALID, can be driven HIGH.

Figure A3.1: Exit from reset

A3.2. Channel handshake

# A3.2 Channel handshake

All AXI channels use the same VALID/READY handshake process to transfer address, data, and control information. This two-way flow control mechanism means both the Manager and Subordinate can control the rate that the information moves between Manager and Subordinate. The source generates the VALID signal to indicate

when the address, data, or control information is available. The destination generates the READY signal to indicate that it can accept the information. Transfer occurs only when both the VALID and READY signals are HIGH.

On Manager and Subordinate interfaces, there must be no combinatorial paths between input and output signals. Figure A3.2 to Figure A3.4 show examples of the handshake process.

The source presents information after edge 1 and asserts the VALID signal as shown in Figure A3.2. The destination asserts the READY signal after edge 2. The source must keep its information stable until the transfer occurs at edge 3, when this assertion is recognized.

Figure A3.2: VALID before READY handshake

A source is not permitted to wait until READY is asserted before asserting VALID.

When VALID is asserted, it must remain asserted until the handshake occurs, at a rising clock edge when VALID and READY are both asserted.

In Figure A3.3 the destination asserts READY after edge 1, before the address, data, or control information is valid. This assertion indicates that it can accept the information. The source presents the information and asserts VALID after edge 2, then the transfer occurs at edge 3, when this assertion is recognized. In this case, transfer occurs in a single cycle.

Figure A3.3: READY before VALID handshake

A destination is permitted to wait for VALID to be asserted before asserting the corresponding READY. If READY is asserted, it is permitted to deassert READY before VALID is asserted.

In Figure A3.4, both the source and destination happen to indicate that they can transfer the address, data, or control information after edge 1. In this case, the transfer occurs at the rising clock edge when the assertion of both VALID and READY can be recognized. These assertions mean that the transfer occurs at edge 2.

A3.2. Channel handshake

0	1	2	3	4

Figure A3.4: VALID with READY handshake

The individual AXI channel handshake mechanisms are described in A3.4 Relationships between the channels.

# A3.3 Write and read channels

This section describes the AXI write and read channels. The channels are:

- A3.3.1 Write request channel (AW)

- A3.3.2 Write data channel (W)

- A3.3.3 Write response channel (B)

- A3.3.4 Read request channel (AR)

- A3.3.5 Read data channel (R)

For interfaces that use A15.3 DVM messages, there are two additional channels:

- A3.6.1 Snoop request channel (AC)

- A3.6.2 Snoop response channel (CR)

## A3.3.4	Read request channel (AR)

The control signals for the read request channel are shown in Table A3.5.

Table A3.5: Read request channel control signals

| Name | Width | Source | Description |
| --- | --- | --- | --- |
| ARVALID | 1 | Manager | Read request valid indicator. |
| ARREADY | 1 | Subordinate | Read request ready indicator. |

The Manager can assert the ARVALID signal only when it drives a valid request. When asserted, ARVALID must remain asserted until the rising clock edge after the Subordinate asserts the ARREADY signal.

The default state of ARREADY can be either HIGH or LOW. It is recommended to use HIGH as the default state for ARREADY. If ARREADY is HIGH, then the Subordinate must be able to accept any valid request that is presented to it.

It is not recommended to default ARREADY LOW because it forces the transfer to take at least two cycles, one to assert ARVALID and another to assert ARREADY.

## A3.3.5	Read data channel (R)

The control signals for the read data channel are shown in Table A3.6.

Table A3.6: Read data channel control signals

| Name | Width | Source | Description |
| --- | --- | --- | --- |
| RVALID | 1 | Subordinate | Read data valid indicator. |
| RREADY | 1 | Manager | Read data ready indicator. |
| RLAST | 1 | Subordinate | Indicates the last read data transfer of a transaction. |

The Subordinate can assert the RVALID signal only when it drives valid signals on the read data channel. When asserted, RVALID must remain asserted until the rising clock edge after the Manager asserts RREADY. Even if a Subordinate has only one source of read data, it must assert the RVALID signal only in response to a request.

The Manager interface uses the RREADY signal to indicate that it accepts the data. The default state of RREADY can be HIGH, but only if the Manager is able to accept read data immediately when it starts a read transaction.

The Subordinate must assert the RLAST signal when it is driving the final read transfer in the transaction. It is recommended that RDATA is driven to zero for inactive byte lanes.

A Manager that does not use RLAST can omit the input from its interface.

The property RLAST_Present is used to determine if the RLAST signal is present.

Table A3.7: RLAST_Present property

| RLAST_Present | Default | Description |
| --- | --- | --- |
| True | Y | RLAST is present. |
| False |  | RLAST is not present. |

A3.4. Relationships between the channels

# A3.4 Relationships between the channels

The AXI protocol requires the following relationships to be maintained:

- A write response must always follow the last write transfer in a write transaction.

- Read data and responses must always follow the read request.

- Channel handshakes must conform to the dependencies defined in A3.5 Dependencies between channel handshake signals.

- When a Manager issues a write request, it must be able to provide all write data for that transaction, without dependency on other transactions from that Manager.

- When a Manager has issued a write request and all write data, it must be able to accept all responses for that transaction, without dependency on other transactions from that Manager.

- When a Manager has issued a read request, it must be able to accept all read data for that transaction, without dependency on other transactions from that Manager.

– Note that a Manager can rely on read data returning in order from transactions that use the same ID, so the Manager only needs enough storage for read data from transactions with different IDs.

- A Manager is permitted to wait for one transaction to complete before issuing another transaction request.

- A Subordinate is permitted to wait for one transaction to complete before accepting or issuing transfers for another transaction.

- A Subordinate must not block acceptance of data-less write requests due to transactions with leading write data.

The protocol does not define any other relationship between the channels.

The lack of relationship means, for example, that the write data can appear at an interface before the write request for the transaction. This can occur if the write request channel contains more register stages than the write data channel. Similarly, the write data might appear in the same cycle as the request.

When the interconnect is required to determine the destination address space or Subordinate space, it must realign the request and write data. This realignment is required to assure that the write data is signaled as being valid only to the Subordinate that it is destined for.

A3.5. Dependencies between channel handshake signals

# A3.5 Dependencies between channel handshake signals

There are dependencies between channels for write, read, and snoop transactions. These are described in the sections below and include dependency diagrams, where:

- Single-headed arrows point to signals that can be asserted before or after the signal at the start of the arrow.

- Double-headed arrows point to signals that must be asserted only after assertion of the signal at the start of the arrow.

## A3.5.1	Write transaction dependencies

For transactions on the write channels, Figure A3.5 shows the handshake signal dependencies. The rules are:

- The Manager must not wait for the Subordinate to assert AWREADY or WREADY before asserting AWVALID or WVALID. This applies to every write data transfer in a transaction.

- The Subordinate can wait for AWVALID or WVALID, or both, before asserting AWREADY.

- The Subordinate can assert AWREADY before AWVALID or WVALID, or both, are asserted.

- The Subordinate can wait for AWVALID or WVALID, or both, before asserting WREADY.

- The Subordinate can assert WREADY before AWVALID or WVALID, or both, are asserted.

- The Subordinate must wait for AWVALID, AWREADY, WVALID, and WREADY to be asserted before asserting BVALID.

- The Subordinate must also wait for WLAST to be asserted before asserting BVALID. This wait is because the write response, BRESP, must be signaled only after the last data transfer of a write transaction.

- The Subordinate must not wait for the Manager to assert BREADY before asserting BVALID.

- The Manager can wait for BVALID before asserting BREADY.

- The Manager can assert BREADY before BVALID is asserted.

### AWREADY	WREADY	BREADY

Figure A3.5: Write transaction handshake dependencies

## A3.5.2	Read transaction dependencies

For transactions on the read channels, Figure A3.6 shows the handshake signal dependencies. The rules are:

- The Manager must not wait for the Subordinate to assert ARREADY before asserting ARVALID.

- The Subordinate can wait for ARVALID to be asserted before it asserts ARREADY.

- The Subordinate can assert ARREADY before ARVALID is asserted.

A3.5. Dependencies between channel handshake signals

- The Subordinate must wait for both ARVALID and ARREADY to be asserted before it asserts RVALID to indicate that valid data is available.

- The Subordinate must not wait for the Manager to assert RREADY before asserting RVALID.

- The Manager can wait for RVALID to be asserted before it asserts RREADY.

- The Manager can assert RREADY before RVALID is asserted.

### ARVALID  RVALID

ARREADY	RREADY

Figure A3.6: Read transaction handshake dependencies

A3.6. Snoop channels

# A3.6 Snoop channels

DVM messages are transported between interconnect and Manager components using snoop channels. When DVM messages are supported, there is a snoop request channel (AC) and snoop response channel (CR).

## A3.6.1	Snoop request channel (AC)

The control signals for the snoop request channel are shown in Table A3.8.

Table A3.8: Snoop request channel control signals

| Name | Width | Source | Description |
| --- | --- | --- | --- |
| ACVALID | 1 | Subordinate | Snoop request valid indicator. |
| ACREADY | 1 | Manager | Snoop request ready indicator. |

A Subordinate can assert the ACVALID signal only when it drives valid address and control information. When asserted, ACVALID must remain asserted until the rising clock edge after the Manager asserts the ACREADY signal.

The default state of ACREADY can be either HIGH or LOW. It is recommended to use HIGH as the default state for ACREADY. If ACREADY is HIGH, then the Manager must be able to accept any valid request that is presented to it.

It is not recommended to default ACREADY LOW because it forces the transfer to take at least two cycles, one to assert ACVALID and another to assert ACREADY.

## A3.6.2	Snoop response channel (CR)

The control signals for the snoop response channel are shown in Table A3.9.

Table A3.9: Snoop response channel control signals

| Name | Width | Source | Description |
| --- | --- | --- | --- |
| CRVALID | 1 | Manager | Snoop response valid indicator. |
| CRREADY | 1 | Subordinate | Snoop response ready indicator. |

The Manager can assert the CRVALID signal only when it drives valid signals on the snoop response channel. When asserted, CRVALID must remain asserted until the rising clock edge after the Subordinate asserts CRREADY.

The Subordinate interface uses the CRREADY signal to indicate that it accepts the response. The default state of CRREADY can be HIGH, but only if the Subordinate is able to accept the snoop response immediately when it starts a snoop transaction.

A3.6. Snoop channels

## A3.6.3	Snoop transaction dependencies

For transactions on the snoop channels, Figure A15.2 shows the handshake signal dependencies. The rules are:

- The Subordinate must not wait for the Manager to assert ACREADY before asserting ACVALID.

- The Manager can wait for ACVALID to be asserted before it asserts ACREADY.

- The Manager can assert ACREADY before ACVALID is asserted.

- The Manager must wait for both ACVALID and ACREADY to be asserted before it asserts CRVALID to indicate that a valid response is available.

- The Manager must not wait for the Subordinate to assert CRREADY before asserting CRVALID.

- The Subordinate can wait for CRVALID to be asserted before it asserts CRREADY.

- The Subordinate can assert CRREADY before CRVALID is asserted.

### ACVALID 	 CRVALID

ACREADY	CRREADY

Figure A3.7: Snoop transaction handshake dependencies