Chapter A4

# AXI Transactions

The AXI protocol uses transactions for communication between Managers and Subordinates. All transactions include a request and a response. Write and read transactions also include one or more data transfers.

This chapter describes the transaction requests, responses, and data transfers. It contains the following sections:

- A4.1 Transaction request

- A4.2 Write and read data

- A4.3 Transaction response

# A4.1 Transaction request

An AXI Manager initiates a transaction by issuing a request to a Subordinate. A request includes transaction attributes and the address of the first data transfer. If the transaction includes more than one data transfer, the Subordinate must calculate the addresses of subsequent transfers.

A transaction must not cross a 4KB address boundary. This prevents a transaction from crossing a boundary between two Subordinates. It also limits the number of address increments that a Subordinate must support.

## A4.1.1	Size attribute

Size indicates the maximum number of bytes in each data transfer.

For read transactions, Size indicates how many data bytes must be valid in each read data transfer.

For write transactions, Size indicates how many data byte lanes are permitted to be active. The write strobes indicate which of those bytes are valid in each transfer.

Size must not exceed the data width of an interface, as determined by the DATA_WIDTH property. If Size is smaller than DATA_WIDTH, a subset of byte lanes is used for each transfer.

Size is communicated using the AWSIZE and ARSIZE signals on the write request and read request channels, respectively. In this specification, AxSIZE indicates AWSIZE and ARSIZE.

Table A4.1: AxSIZE signals

| Name | Width | Default | Description |
| --- | --- | --- | --- |
| AWSIZE, ARSIZE | 3 | DATA_WIDTH/8 | Indicates the maximum number of bytes in each data transfer within a transaction. |

Size is encoded on the AxSIZE signals as shown in Table A4.2.

Table A4.2: AxSIZE encodings

| AxSIZE | Label | Meaning |
| --- | --- | --- |
| 0b000 | 1 | 1 byte per transfer |
| 0b001 | 2 | 2 bytes per transfer |
| 0b010 | 4 | 4 bytes per transfer |
| 0b011 | 8 | 8 bytes per transfer |
| 0b100 | 16 | 16 bytes per transfer |
| 0b101 | 32 | 32 bytes per transfer |
| 0b110 | 64 | 64 bytes per transfer |
| 0b111 | 128 | 128 bytes per transfer |

The property SIZE_Present is used to determine if the AxSIZE signals are present.

Table A4.3: SIZE_Present property

| SIZE_Present | Default | Description |
| --- | --- | --- |
| True | Y | AWSIZE and ARSIZE are present. |
| False |  | AWSIZE and ARSIZE are not present. |

A Manager that only issues requests of full data width can omit the AxSIZE outputs from its interface. An attached Subordinate must have its AxSIZE input tied according to the data width.

## A4.1.2	Length attribute

The Length attribute defines the number of data transfers in a transaction.

Size x Length is the maximum number of bytes that can be transferred in a transaction. If the address is unaligned or there are deasserted write strobes, the actual number of bytes transferred can be lower than Size x Length.

A Manager must issue the number of write data transfers according to Length.

A Subordinate must issue the number of read data transfers according to Length.

Length is communicated using the AWLEN and ARLEN signals on the write request and read request channels, respectively. In this specification, AxLEN indicates AWLEN and ARLEN.

Table A4.4: AxLEN signals

| Name | Width | Default | Description |
| --- | --- | --- | --- |
| AWLEN, ARLEN | 8 | 0x00 | The total number of transfers in a transaction, encoded as: Length = AxLEN + 1. |

The property LEN_Present is used to determine if the signals are present. Table A4.5 shows the legal values of LEN_Present.

Table A4.5: LEN_Present property

| LEN_Present | Default | Description |
| --- | --- | --- |

| True | Y | AWLEN and ARLEN are present. |
| --- | --- | --- |
| False |  | AWLEN and ARLEN are not present. |

A Manager that only issues requests of Length 1 can omit the AxLEN outputs from its interface. An attached Subordinate must have its AxLEN input tied to 0x00.

The following rules apply to transaction Length:

- For wrapping bursts, Length can be 2, 4, 8, or 16.

- For fixed bursts, Length can be up to 16.

- A transaction must not cross a 4KB address boundary.

- Early termination of transactions is not supported.

No component can terminate a transaction early. However, to reduce the number of data transfers in a write transaction, the Manager can disable further writing by deasserting all the write strobes. In this case, the Manager must complete the remaining transfers in the transaction. In a read transaction, the Manager can discard read data, but it must complete all transfers in the transaction.

## A4.1.3	Maximum number of bytes in a transaction

The maximum number of bytes in a transaction is 4KB and transactions are not permitted to cross a 4KB boundary. However, many Managers generate transactions which are always smaller than this.

A Subordinate or interconnect might benefit from this information. For example, a Subordinate might be able to optimize away some decode logic. An interconnect striping at a granule smaller than 4KB might be able to avoid burst splitting if it knows that transactions will not cross the stripe boundary.

The property Max_Transaction_Bytes defines the maximum size of a transaction in bytes as shown in Table A4.6.

Table A4.6: Max_Transaction_Bytes property

| Name | Values | Default | Description |
| --- | --- | --- | --- |
| Max_Transaction_Bytes | 64, 128, 256, 512, 1024, 2048, 4096 | 4096 | A Manager issues transactions where Size x Length is Max_Transaction_Bytes or smaller and transactions do not cross a Max_Transaction_Bytes boundary. A Subordinate can only accept transactions where Size x Length is Max_Transaction_Bytes or smaller. |

When connecting Manager and Subordinate interfaces, Table A4.7 indicates combinations of Max_Transaction_Bytes that are compatible.

Table A4.7: Max_Transaction_Bytes interoperability

| Manager < Subordinate | Manager == Subordinate | Manager > Subordinate |
| --- | --- | --- |
| Compatible | Compatible | Not compatible |

## A4.1.4	Burst attribute

The Burst attribute describes how the address increments between transfers in a transaction. There are three different Burst types:

FIXED

This Burst type is used for repeated accesses to the same location such as when loading or emptying a FIFO.

- The address is the same for every transfer in the burst.

- The byte lanes that are valid are constant for all transfers. However, within those byte lanes, the actual bytes that have WSTRB asserted can differ for each transfer.

- The Length of the burst can be up to 16 transfers.

- The FIXED burst type can only be used with WriteNoSnoop or ReadNoSnoop Opcodes. See Chapter A8

Request Opcodes for more information.

INCR (incrementing)

With this Burst type, the address for each transfer is an increment of the address for the previous transfer. The increment value depends on the transaction Size. For example, for an aligned start address, the address for each transfer in a transaction with a Size of 4 bytes is the previous address plus 4. This Burst type is used for accesses to normal sequential memory.

WRAP (wrapping)

This Burst type is similar to INCR except that the address wraps around to a lower address if an upper address limit is reached. The following restrictions apply:

- The start address must be aligned to the size of each transfer.

- The Length of the burst must be 2, 4, 8, or 16 transfers. The behavior of a wrapping transaction is:

- The lowest address accessed by the transaction is the start address aligned to the total size of the data to be transferred, that is Size * Length. This address is defined as the wrap boundary.

- After each transfer, the address increments in the same way as for an INCR burst. However, if this incremented address is ((wrap boundary)+ (Size * Length)), then the address wraps round to the

wrap boundary.

- The first transfer in the transaction can use an address that is higher than the wrap boundary, subject to the restrictions that apply to wrapping transactions. The address wraps when the first address is higher than the

wrap boundary. This Burst type is used for cache line accesses.

Burst is communicated using the AWBURST and ARBURST signals on the write request and read request channels, respectively. In this specification, AxBURST indicates AWBURST and ARBURST.

Table A4.8: AxBURST signals

| Name | Width | Default | Description |
| --- | --- | --- | --- |
| AWBURST, ARBURST | 2 | 0b01 (INCR) | Describes how the address increments between transfers in a transaction. |

Burst is encoded on the AxBURST signals as shown in Table A4.9.

Table A4.9: AxBURST encodings

| AxBURST | Label | Meaning |
| --- | --- | --- |

| 0b00 | FIXED | Fixed burst |
| --- | --- | --- |
| 0b01 | INCR | Incrementing burst |
| 0b10 | WRAP | Wrapping burst |
| 0b11 | RESERVED | - |

The property BURST_Present is used to determine if the AxBURST signals are present.

A Manager that only issues requests with a Burst type of INCR can omit the AxBURST outputs from its interface. An attached Subordinate must have its AxBURST input tied to 0b01.

Table A4.10: BURST_Present property

| BURST_Present | Default | Description |
| --- | --- | --- |
| True | Y | AWBURST and ARBURST are present. |

False	AWBURST and ARBURST are not present.

Fixed_Burst_Disable

A Burst type of FIXED is not commonly used, and a property Fixed_Burst_Disable is defined in Table A4.11 to indicate if a component supports it.

Table A4.11: Fixed_Burst_Disable property

| Fixed_Burst_Disable | Default | Description |
| --- | --- | --- |
| True |  | Requests with Burst type FIXED are not supported by a Subordinate interface and not generated by a Manager interface. |
| False | Y | Requests with Burst type FIXED are supported by a Subordinate interface and might be generated by a Manager interface. |

Compatibility between Manager and Subordinate interfaces, according to the values of the Fixed_Burst_Disable property is shown in Table A4.12.

Table A4.12: Fixed_Burst_Disable compatibility

| Fixed_Burst_Disable | Subordinate: False | Subordinate: True |
| --- | --- | --- |
| Manager: False | Compatible. | Not compatible. |
| Manager: True | Compatible. | Compatible. |

## A4.1.5	Transfer address

This section provides methods for determining the address and byte lanes of transfers within a transaction. The start address for a transaction is indicated using the AxADDR signals.

Table A4.13: AxADDR signals

| Name | Width |  | Default | Description |
| --- | --- | --- | --- | --- |
| AWADDR, ARADDR | ADDR_WIDTH |  | - | Address of first transfer in a transaction. |
| Address width The property ADDR_WIDTH is used to define the address width. |  |  |  | Table A4.14: ADDR_WIDTH property |
| Name | Values | Default | Description |  |
| ADDR_WIDTH | 1..64 | 32 | Width of AWADDR, ARADDR, and ACADDR in bits. |  |

The protocol supports communication between components that have different physical address space sizes. Components with different physical address space sizes must communicate as follows:

- The component with the smaller physical address space must be positioned within an aligned window in the larger physical address space. Typically, the window is located at the bottom of the larger physical address space. However, it is acceptable for the component with the smaller physical address space to be positioned in an offset window within the larger physical address space.

- An outgoing transaction must have the required additional higher-order bits added to the transaction address.

- An incoming transaction must be examined so that:

  - A transaction that is within the address window has the higher-order address bits removed and is passed through.

  - A transaction that does not have the required higher-order address bits is suppressed.

It is the responsibility of the interconnect to provide the required functionality.

## A4.1.6	Transaction equations

The equations listed here are used to determine the address and active data byte lanes for each transfer in a transaction. The equations use the following additional variables:

- Start_Addr: The start address that is issued by the Manager.

- Data_Bytes: The width of the data channels in bytes (DATA_WIDTH/8).

- Aligned_Addr: The aligned version of the start address.

- Address_N: The address of transfer N in a transaction. N is 1 for the first transfer in a transaction.

- Wrap_Boundary: The lowest address within a wrapping transaction.

- Lower_Byte_Lane: The byte lane of the lowest addressed byte of a transfer.

- Upper_Byte_Lane: The byte lane of the highest addressed byte of a transfer.

- INT(x): The rounded-down integer value of x.

These equations determine addresses of transfers within a burst:

Start_Addr = AxADDR

Aligned_Addr = INT(Start_Addr / Size)* Size

This equation determines the address of the first transfer in a burst:

Address_1 = Start_Addr

For an INCR burst and for a WRAP burst for which the address has not wrapped, this equation determines the address of any transfer after the first transfer in a burst:

Address_N = Aligned_Addr + (N - 1)* Size

For a WRAP burst, the Wrap_Boundary variable defines the wrapping boundary:

Wrap_Boundary = INT(Start_Addr / (Size * Length))* Size * Length

For a WRAP burst, if Address_N = Wrap_Boundary + Size * Length, then:

- Use this equation for the current transfer:

Address_N = Wrap_Boundary

- Use this equation for any subsequent transfers:

Address_N = Start_Addr + ((N - 1)* Size)- (Size * Length)

These equations determine the byte lanes to use for the first transfer in a burst:

Lower_Byte_Lane = Start_Addr - (INT(Start_Addr/Data_Bytes)* Data_Bytes) Upper_Byte_Lane = Aligned_Addr + (Size-1)- (INT(Start_Addr/Data_Bytes)* Data_Bytes)

These equations determine the byte lanes to use for all transfers after the first transfer in a burst:

Lower_Byte_Lane = Address_N - (INT(Address_N / Data_Bytes)* Data_Bytes) Upper_Byte_Lane = Lower_Byte_Lane + Size - 1

Data is transferred on:

DATA((8 * Upper_Byte_Lane)+ 7: (8 * Lower_Byte_Lane))

The transaction container describes all the bytes that could be accessed in that transaction, if the address is aligned and strobes are asserted:

Container_Size = Size * Length

For INCR bursts:

Container_Lower = Aligned_Addr

Container_Upper = Aligned_Addr + Container_Size For WRAP bursts:

Container_Lower = Wrap_Boundary

Container_Upper = Wrap_Boundary + Container_Size

## A4.1.7	Pseudocode description of the transfers

// DataTransfer()

DataTransfer(Start_Addr, Size, Length, Data_Bytes, Burst, IsWrite)

// IsWrite is TRUE for a write, and FALSE for a read

addr = Start_Addr; // Variable for current address Aligned_Addr = (INT(addr/Size) * Size);

aligned = (Aligned_Addr == addr); // Check whether addr aligned to Size Container_Size = Size * Length;

if Burst == WRAP then

Lower_Wrap_Boundary = (INT(addr/Container_Size) * Container_Size);

// addr must be aligned for a wrapping burst Upper_Wrap_Boundary = Lower_Wrap_Boundary + Container_Size;

for n = 1 to Length

Lower_Byte_Lane = addr - (INT(addr/Data_Bytes) * Data_Bytes); if aligned then Upper_Byte_Lane = Lower_Byte_Lane + Size - 1 else

Upper_Byte_Lane = Aligned_Addr + Size - 1

- (INT(addr/Data_Bytes) * Data_Bytes);

// Perform data transfer if IsWrite then dwrite(addr, Lower_Byte_Lane, Upper_Byte_Lane)

else dread(addr, Lower_Byte_Lane, Upper_Byte_Lane);

// Increment address if necessary if Burst != FIXED then if aligned then addr = addr + Size; if Burst == WRAP then if addr >= Upper_Wrap_Boundary then addr = Lower_Wrap_Boundary; else addr = Aligned_Addr + Size; aligned = TRUE; // All transfers after the first are aligned

return;

## A4.1.8	Regular transactions

There are many options of burst, size, and length for a transaction. However, some interfaces and transaction types might only use a subset of these options. If a Subordinate component is attached to a Manager which uses only a subset of transaction options, it can be designed with simplified decode logic.

The Regular attribute is defined, to identify transactions which meet the following criteria:

- Length is 1, 2, 4, 8, or 16 transfers.

- Size is the same as the data channel width if Length is greater than 1.

- Burst is INCR or WRAP, not FIXED.

- Address is aligned to the transaction container for INCR transactions.

- Address is aligned to Size for WRAP transactions.

The Regular_Transactions_Only property is used to define whether a Manager issues only Regular type transactions and if a Subordinate only supports Regular transactions.

Table A4.15: Regular_Transactions_Only property

| Regular_Transactions_Only | Default | Description |
| --- | --- | --- |
| True |  | Only Regular transactions are issued/supported. |
| False | Y | All legal combinations of Burst, Size, and Length are issued/supported. |

Interoperability rules for Regular transactions are shown in Table A4.16.

Table A4.16: Regular_Transactions_Only interoperability

|  | Subordinate: False | Subordinate: True |
| --- | --- | --- |
| Manager: False | Compatible. | Not compatible. If the Manager issues a transaction that is not Regular, then data corruption or deadlock might occur. |
| Manager: True | Compatible. | Compatible. |

# A4.2 Write and read data

This section describes the transfers of varying sizes on the AXI write and read data channels and how the interface performs mixed-endian and unaligned transfers.

## A4.2.1	Write strobes

The WSTRB signal carries write strobes that specify which byte lanes of the write data channel contain valid information.

Table A4.17: WSTRB signal

| Name | Width | Default | Description |
| --- | --- | --- | --- |
| WSTRB | DATA_WIDTH / 8 | All ones | Indicates which byte lanes of WDATA contain valid data in a write transaction. |

There is one write strobe for each 8 bits of the write data channel, therefore WSTRB[n] corresponds to WDATA[(8n)+7:(8n)].

When WVALID is HIGH:

- Data bytes to be written have a corresponding write strobe set HIGH.

- Inside of the transaction container any number of write strobes can be HIGH. If all write strobes are LOW, no data is written from that transfer.

- Outside of the transaction container all write strobes must be LOW.

When WVALID is LOW, the write strobes can take any value, although it is recommended that they are either driven LOW or held at their previous value.

It is recommended that WDATA is driven to zero for byte lanes where the strobe is LOW.

The property WSTRB_Present is used to indicate if the WSTRB signal is present on an interface.

Table A4.18: WSTRB_Present property

| WSTRB_Present | Default | Description |
| --- | --- | --- |
| True | Y | WSTRB is present. |
| False |  | WSTRB is not present. |

A Manager that only issues transactions where all write strobes are asserted can omit the WSTRB output from its interface. An attached Subordinate must have its WSTRB input tied HIGH.

## A4.2.2	Narrow transfers

When a Manager generates a transfer that is narrower than its data channel, the address and control information determine the byte lanes that the transfer uses:

- When Burst is INCR or WRAP, different byte lanes are used for each data transfer in the transaction.

- When Burst is FIXED, the same byte lanes are used for each data transfer in the transaction.

Two examples of byte lane use are shown in Figure A4.1 and Figure A4.2. The shaded cells indicate bytes that are not transferred.

In Figure A4.1:

- The transaction has five data transfers.

- The starting address is 0.

- Each transfer is 8 bits.

- The transfers are on a 32-bit data channel.

- The burst type is INCR.

| 31 24 | 23 16 | 15 8 | 7 | 0 |
| --- | --- | --- | --- | --- |
|  |  |  | D[7:0] |  |
|  |  | D[15:8] |  |  |
|  | D[23:16] |  |  |  |
| D[31:24] |  |  |  |  |
|  |  |  | D[7:0] |  |

1st transfer 2nd transfer 3rd transfer 4th transfer 5th transfer

WDATA[31:0]

Figure A4.1: Narrow transfer example with 8-bit transfers

In Figure A4.2:

- The transaction has three data transfers.

- The starting address is 4.

- Each transfer is 32 bits.

- The transfers are on a 64-bit data channel.

| 63 | 56 | 55 48 | 47 | 40 | 39 32 | 3 1 | 24 | 2 3 | 16 | 15 | 8 | 7 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D[63:56] |  | D[55:48] | D[47:40] |  | D[39:32] |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | D[31:24] |  | D[23:16] |  | D[15:8] |  | D[7:0] |  |
| D[63:56] |  | D[55:48] | D[47:40] |  | D[39:32] |  |  |  |  |  |  |  |  |

1st transfer

2nd transfer 3rd transfer

WDATA[63:0]

Figure A4.2: Narrow transfer example with 32-bit transfers

## A4.2.3	Byte invariance

To access mixed-endian data structures in a single memory space, the AXI protocol uses a byte-invariant endianness scheme.

Byte-invariant endianness means that for any multi-byte element in a data structure:

- The element uses the same continuous bytes of memory, regardless of the endianness of the data.

- The endianness determines the order of those bytes in memory, meaning it determines whether the first byte in memory is the most significant byte (MSB) or the least significant byte (LSB) of the element.

- Any byte transfer to an address passes the 8 bits of data on the same data channel wires to the same address location, regardless of the endianness of any larger data element that it is a constituent of.

Components that have only one transfer width must have their byte lanes connected to the appropriate byte lanes of the data channel. Components that support multiple transfer widths might require a more complex interface to convert an interface that is not naturally byte-invariant.

Most little-endian components can connect directly to a byte-invariant interface. Components that support only big-endian transfers require a conversion function for byte-invariant operation.

The examples in Figure A4.3 and Figure A4.4 show a 32-bit number 0x0A0B0C0D, stored in a register and in a memory.

In Figure A4.3 there is an example of the big-endian, byte-invariant data structure. In this structure:

- The MSB of the data, which is 0x0A, is stored in the MSB position in the register.

- The MSB of the data is stored in the memory location with the lowest address.

- The other data bytes are positioned in decreasing order of significance. Addr

Addr+1

Addr+2

Memory

| 0x 0 A |  |  |  |  |
| --- | --- | --- | --- | --- |
| 0x 0 B |  |  |  |  |
| 0x 0 C |  |  |  |  |
| 0x 0 D |  |  |  |  |
| 3 1 | 2 4 | 2 3 | 1 6 | 1 5 |
| 0x0 A |  | 0x0 B |  | 0x0 C |

Register

Addr+3

Figure A4.3: Example big-endian byte-invariant data structure

In Figure A4.4 there is an example of a little-endian, byte-invariant data structure. In this structure:

- The LSB of the data, which is 0x0D, is stored in the LSB position in the register.

- The LSB of the data is stored in the memory location with the lowest address.

- The other data bytes are positioned in increasing order of significance.

| 0x 0 D |  |  |  |  |
| --- | --- | --- | --- | --- |
| 0x 0 C |  |  |  |  |
| 0x 0 B |  |  |  |  |
| 0x 0 A |  |  |  |  |
| 3 1 | 2 4 | 2 3 | 1 6 | 1 5 |
| 0x0 A |  | 0x0 B |  | 0x0 |

Addr

Addr+1

Addr+2

Memory

Register

Addr+3

Figure A4.4: Example little-endian byte-invariant data structure

The examples in Figure A4.3 and Figure A4.4 show that byte invariance ensures that big-endian and little-endian structures can coexist in a single memory space without corruption.

In Figure A4.5 there is an example of a data structure that requires byte-invariant access. In this example, the header fields use little-endian ordering, and the payload uses big-endian ordering.

| 31 | 24 | 23 16 | 15 | 8 | 7 | 0 |
| --- | --- | --- | --- | --- | --- | --- |
| D0† |  | Source |  |  | Packet |  |
| Checksum |  |  |  |  | D1† |  |
|  |  |  | Data items |  |  |  |

Header, little-endian byte ordering

Payload, big-endian byte ordering

Payload

Payload

Payload

Payload

† 16-bit continuous Destination field

Figure A4.5: Example

structure

mixed-endian data

In this	example structure, Data

items is a	two-byte little-endian

element,	meaning its lowest

address is its LSB. The use of byte invariance ensures that a big-endian access to the payload does not corrupt the little-endian element.

## A4.2.4	Unaligned transfers

AXI supports unaligned transfers. For any transaction that is made up of data transfers wider than 1 byte, the first bytes accessed might be unaligned with the natural address boundary. For example, a 32-bit data packet that starts at a byte address of 0x1002 is not aligned to the natural 32-bit address boundary.

A Manager can:

- Use the low-order address lines to signal an unaligned start address.

- Provide an aligned address and use the byte lane strobes to signal the unaligned start address.

The information on the low-order address lines must be consistent with the information on the byte lane strobes. The Subordinate is not required to take special action based on any alignment information from the Manager.

In Figure A4.6 there are examples of aligned and unaligned 32-bit transactions on a 32-bit data channel. Each row in the figure represents a transfer and the shaded cells indicate bytes that are not transferred.

Address: 0x002nd transfer Transfer size: 32-bits

Burst type: incrementing Burst length: 4 transfers

31	24  23	16 15	8 7	0

| 0x03 | 0x02 | 0x01 | 0x00 |
| --- | --- | --- | --- |
| 0x07 | 0x06 | 0x05 | 0x04 |
| 0x0B | 0x0A | 0x09 | 0x08 |
| 0x0F | 0x0E | 0x0D | 0x0C |

WDATA[31:0]

1st transfer

3rd transfer

4th transfer

Address: 0x012nd transfer Transfer size: 32-bits

Burst type: incrementing Burst length: 4 transfers

31	24  23	16 15	8 7	0

| 0x03 | 0x02 | 0x01 | 0x00 |
| --- | --- | --- | --- |
| 0x07 | 0x06 | 0x05 | 0x04 |
| 0x0B | 0x0A | 0x09 | 0x08 |
| 0x0F | 0x0E | 0x0D | 0x0C |

WDATA[31:0]

31	24 23	16 15	8  7	0

1st transfer

3rd transfer

4th transfer

1st transfer

| 0x03 | 0x02 | 0x01 | 0x00 |
| --- | --- | --- | --- |
| 0x07 | 0x06 | 0x05 | 0x04 |
| 0x0B | 0x0A | 0x09 | 0x08 |
| 0x0F | 0x0E | 0x0D | 0x0C |
| 0x13 | 0x12 | 0x11 | 0x10 |

2nd transfer Address: 0x01 Transfer size: 32-bits3rd transfer

Burst type: incrementing Burst length: 5 transfers

4th transfer 5th transfer

WDATA[31:0]

31	24  23	16 15	8 7	0

| 0x07 | 0x06 | 0x05 | 0x04 |
| --- | --- | --- | --- |
| 0x0B | 0x0A | 0x09 | 0x08 |
| 0x0F | 0x0E | 0x0D | 0x0C |
| 0x13 | 0x12 | 0x11 | 0x10 |
| 0x17 | 0x16 | 0x15 | 0x14 |

1st transfer

2nd transfer Address: 0x07 Transfer size: 32-bits

Burst type: incrementing3rd transfer

Burst length: 5 transfers

4th transfer 5th transfer

WDATA[31:0]

Figure A4.6: Aligned and unaligned transfers on a 32-bit data channel

In Figure A4.7 there are examples of aligned and unaligned 32-bit transactions on a 64-bit data channel. Each row in the figure represents a transfer and the shaded cells indicate bytes that are not transferred.

Address: 0x002nd transfer Transfer size: 32-bits

Burst type: incrementing Burst length: 4 transfers

63	56 55	48 47	40 39	32 31	24 23	16 15	8 7	0

| 0x07 | 0x06 | 0x05 | 0x04 | 0x03 | 0x02 | 0x01 | 0x00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x07 | 0x06 | 0x05 | 0x04 | 0x03 | 0x02 | 0x01 | 0x00 |
| 0x0F | 0x0E | 0x0D | 0x0C | 0x0B | 0x0A | 0x09 | 0x08 |
| 0x0F | 0x0E | 0x0D | 0x0C | 0x0B | 0x0A | 0x09 | 0x08 |

WDATA[63:0]

63	56 55	48 47	40 39	32 31	24 23	16 15	8 7	0

1st transfer

3rd transfer

4th transfer

| 0x07 | 0x06 | 0x05 | 0x04 | 0x03 | 0x02 | 0x01 | 0x00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x0F | 0x0E | 0x0D | 0x0C | 0x0B | 0x0A | 0x09 | 0x08 |

1st transfer

Address: 0x072nd transfer Transfer size: 32-bits

Burst type: incrementing Burst length: 4 transfers

| 0x0F | 0x0E | 0x0D | 0x0C | 0x0B | 0x0A | 0x09 | 0x08 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x17 | 0x16 | 0x15 | 0x14 | 0x13 | 0x12 | 0x11 | 0x10 |

3rd transfer

4th transfer

WDATA[63:0]

2nd transfer Address: 0x07 Transfer size: 32-bits3rd transfer

Burst type: incrementing Burst length: 5 transfers

63	56 55	48 47	40 39	32 31	24 23	16 15	8 7	0

| 0x07 | 0x06 | 0x05 | 0x04 | 0x03 | 0x02 | 0x01 | 0x00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x0F | 0x0E | 0x0D | 0x0C | 0x0B | 0x0A | 0x09 | 0x08 |
| 0x0F | 0x0E | 0x0D | 0x0C | 0x0B | 0x0A | 0x09 | 0x08 |
| 0x17 | 0x16 | 0x15 | 0x14 | 0x13 | 0x12 | 0x11 | 0x10 |
| 0x17 | 0x16 | 0x15 | 0x14 | 0x13 | 0x12 | 0x11 | 0x10 |

WDATA[63:0]

1st transfer

4th transfer 5th transfer

Figure A4.7: Aligned and unaligned transfers on a 64-bit data channel

In Figure A4.8 there is an example of an aligned 32-bit wrapping transaction on a 64-bit data channel. Each row in the figure represents a transfer and the shaded cells indicate bytes that are not transferred.

63	56 55	48 47	40 39	32 31	24 23	16 15	8 7	0

| 0x07 | 0x06 | 0x05 | 0x04 | 0x03 | 0x02 | 0x01 | 0x00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x0F | 0x0E | 0x0D | 0x0C | 0x0B | 0x0A | 0x09 | 0x08 |
| 0x0F | 0x0E | 0x0D | 0x0C | 0x0B | 0x0A | 0x09 | 0x08 |

1st transfer

| 0x07 | 0x06 | 0x05 | 0x04 | 0x03 | 0x02 | 0x01 | 0x00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Address: 0x042nd transfer Transfer size: 32-bits Burst type: wrapping

Burst length: 4 transfers

3rd transfer

4th transfer

WDATA[63:0]

Figure A4.8: Aligned wrapping transfers on a 64-bit channel

# A4.3 Transaction response

Every AXI transaction includes one or more response transfers sent by the Subordinate to indicate the result of the transaction.

Transactions on the write channels have one or more write responses. Transactions on the read channels have one or more read responses.

Atomic transactions have write and read responses, see A7.4 Atomic transactions.

## A4.3.1	Write response

Write responses are transported using the BRESP signal on the write response channel. All transactions on the write channels have one Completion response which indicates the result of the transaction. Some transactions also have a second write response, for example to indicate Persistence, see A10.8.4 PCMO response on the B channel. The BRESP and BCOMP signals are used to send write responses.

Table A4.19: BRESP and BCOMP signals

| Name | Width | Default | Description |
| --- | --- | --- | --- |
| BRESP | BRESP_WIDTH | 0b000 (OKAY) | Indicates the result of a transaction that uses the write channels. |
| BCOMP | 1 | 0b1 | Asserted HIGH to indicate a Completion response. |

The BRESP_WIDTH property is defined in Table A4.20.

Table A4.20: BRESP_WIDTH property

| Name | Values | Default | Description |
| --- | --- | --- | --- |
| BRESP_WIDTH | 0, 2, 3 | 2 | Width of BRESP in bits. Must be 3 if: Untranslated_Transactions = v2 OR Untranslated_Transactions = v3 OR WriteDeferrable_Transaction = True |

BRESP is an optional signal. If the BRESP_WIDTH property is 0, it is not present and assumed to be 0b000

(OKAY).

BCOMP is only present if an interface is using a feature that can have two write responses, these are:

- Cache maintenance for Persistence, see A10.8 CMOs for Persistence.

- Memory Tagging, see A13.2 Memory Tagging Extension (MTE).

If BCOMP is present, it must be asserted for one response transfer of every transaction on the write channels.

The BRESP encodings are shown in Table A4.21.

Table A4.21: BRESP encodings

| BRESP | Label | Meaning |
| --- | --- | --- |

| 0b000 | OKAY | Non-exclusive write: The transaction was successful. If the transaction includes write data, the updated value is observable. Exclusive write: Failed to update the location. |
| --- | --- | --- |
| 0b001 | EXOKAY | Exclusive write succeeded. This response is only permitted for an exclusive write. |
| 0b010 | SLVERR | The request has reached an end point but has not completed successfully. The location might not be fully updated. Typically used when there is a problem within a Subordinate such as trying to access a read-only or powered-down function. |
| 0b011 | DECERR | The request has not reached a point where data can be written. The location might not be fully updated. Typically used when the address decodes to an invalid address. |
| 0b100 | DEFER | Write was unsuccessful because it cannot be serviced at this time. The location is not updated. This response is only permitted for a WriteDeferrable transaction. |
| 0b101 | TRANSFAULT | Write was terminated because of a translation fault which might be resolved by a PRI request. |
| 0b110 | RESERVED | - |
| 0b111 | UNSUPPORTED | Write was unsuccessful because the transaction type is not supported by the target. The location is not updated. This response is only permitted for a WriteDeferrable transaction. |

## A4.3.2	Read response

The read response indicates if the read was successful and whether the data in that transfer is valid.

Read responses are transported using the RRESP signal on the read data channel. There is a read response with every read data transfer in a transaction. The response value is not required to be the same for every read data transfer in a transaction.

It is required that all data transfers as indicated by Length are always completed irrespective of the response. For some responses, the data in that transfer is not required to be valid.

The RRESP signal is defined in Table A4.22.

Table A4.22: RRESP signal

| Name | Width | Default | Description |
| --- | --- | --- | --- |
| RRESP | RRESP_WIDTH | 0b000 (OKAY) | Response for transactions on the read channels. Must be valid when RVALID is asserted. |

The RRESP_WIDTH property is defined in Table A4.23.

Table A4.23: RRESP_WIDTH property

| Name | Values | Default | Description |
| --- | --- | --- | --- |

| RRESP_WIDTH | 0, 2, 3 | 2 | Width of RRESP in bits. Must be 3 if Prefetch_Transaction = True OR Untranslated_Transactions = v2 OR Untranslated_Transactions = v3 OR Shareable_Cache_Support = True |
| --- | --- | --- | --- |

RRESP is an optional signal. If the RRESP_WIDTH property is 0, it is not present and assumed to be 0b000

(OKAY).

The RRESP encodings are shown in Table A4.24.

For responses where data is not required to be valid, the Manager might still sample the RDATA value so the Subordinate should not rely on the response to hide sensitive data.

Table A4.24: RRESP encodings

| RRESP | Label | Meaning |
| --- | --- | --- |
| 0b000 | OKAY | Non-exclusive read: Transaction has completed successfully, read data is valid. Exclusive read: Subordinate does not support exclusive accesses. |
| 0b001 | EXOKAY | Exclusive read succeeded. This response is only permitted for an exclusive read. |
| 0b010 | SLVERR | Transaction has encountered a contained error; only this location is affected. Typically used when there is a problem within a Subordinate such as a FIFO overrun, unsupported transfer size or trying to access a powered-down function. Read data is not valid. |
| 0b011 | DECERR | Transaction has encountered a non-contained error; other locations may be affected. Typically used when the address decodes to an invalid address. Read data is not valid. |
| 0b100 | PREFETCHED | Read data is valid and has been sourced from a prefetched value. |
| 0b101 | TRANSFAULT | Transaction was terminated because of a translation fault which might be resolved by a PRI request. Read data is not valid. |
| 0b110 | OKAYDIRTY | Read data is valid and is Dirty with respect to the value in memory. Only permitted for a response to a ReadShared request. |
| 0b111 | RESERVED | - |

The value of RRESP is not constrained to be the same for every transfer in a transaction. A response of DECERR is generally used when there is a problem accessing a Subordinate, and in this case DECERR is signaled consistently in every transfer of read data. There may be a benefit if a Manager can inspect just one read data transfer to determine whether a DECERR has occurred.

The Consistent_DECERR property is used to define whether a Subordinate signals DECERR consistently within a transaction as shown in Table A4.25.

Table A4.25: Consistent_DECERR property

| Consistent_DECERR | Default | Description |
| --- | --- | --- |

| True |  | DECERR is signaled for every read data transfer, or no read data transfers in each cache line of data. For example, a transaction which crosses a cache line boundary can receive a DECERR response for every read data transfer on one cache line and no data transfers on the next cache line. |
| --- | --- | --- |
| False | Y | DECERR may be signaled on any number of read data transfers. |

A Subordinate interface that does not use the DECERR response can set the Consistent_DECERR property to True.

A Manager with Consistent_DECERR set True can inspect a single data transfer to determine whether a DECERR has occurred.

Setting this property to True can be helpful when bridging between AXI and CHI where DECERR translates to a Non-data Error.

When connecting Manager and Subordinate interfaces, Table A4.26 indicates combinations of Consistent_DECERR that are compatible.

Table A4.26: Consistent_DECERR interoperability

|  | Subordinate: False | Subordinate: True |
| --- | --- | --- |
| Manager: False | Compatible. | Compatible. |
| Manager: True | Not compatible. A DECERR response might be missed by the Manager. | Compatible. |

## A4.3.3	Subordinate Busy indicator

When providing a response, a Subordinate can indicate its current level of activity using the Busy indicator. This information can be used to control the issue rate of a Manager or how many speculative transactions it produces.

The Busy indication is useful for components with a shared resource, such as a memory controller or system cache.

For example, the Busy indication can indicate:

- The level of a shared queue.

- The level of a read or write request queue, depending on the direction of the transaction.

- When the resource usage by a component is more or less than its allocated value.

The Busy_Support property as shown in Table A4.27 is used to define whether an interface includes the Busy indicator signals.

Table A4.27: Busy_Support property

| Busy_Support | Default | Description |
| --- | --- | --- |
| True |  | Subordinate busy is supported. |
| False | Y | Subordinate busy is not supported. |

When Busy_Support is True, the following signals are included on an interface.

Table A4.28: Busy indicator signals

| Name | Width | Default | Description |
| --- | --- | --- | --- |
| BBUSY, RBUSY | 2 | 0b00 | Indicates the current level of Subordinate activity in a transaction response. The value increases as the Subordinate becomes busier. |

For transactions with multiple read data transfers, Busy must be valid but can take a different value for every transfer.

For transactions with multiple write responses, Busy must be valid in the response with BCOMP asserted. For other write responses, Busy is not-applicable and can take any value.

For Atomic transactions with write and read responses, BBUSY and RBUSY are expected, but not required to have the same value.

The exact usage of Busy indicator values is IMPLEMENTATION DEFINED, in Table A4.29 there is an example of how it can be used. In this example, a default value of 0b01 would be appropriate if a Subordinate was not able to generate a dynamic busy indicator.

Table A4.29: Example usage of the Busy indicator

| Busy indicator value | Meaning | Manager behavior |
| --- | --- | --- |
| 0b00 | Not busy | Increase speculative requests |
| 0b01 | Optimally busy | No change |
| 0b10 | Quite busy | Decrease speculative requests |
| 0b11 | Very busy | Heavily decrease speculative requests |

When connecting Manager and Subordinate interfaces, Table A4.30 indicates combinations of Busy_Support that are compatible.

Table A4.30: Busy_Support interoperability

|  | Subordinate: False | Subordinate: True |
| --- | --- | --- |
| Manager: False | Compatible. | Compatible, BUSY outputs are left unconnected. |
| Manager: True | Compatible, BUSY inputs are tied to the default value. | Compatible. |