# Functional Specification for Multi Core Matrix Compute Unit.

Author: Godwin Maben

Date: 5/12/2025

1. Overview

This document specifies a small compute unit designed to perform matrix operations (multiplication, addition, subtraction, and division) based on assembly code instructions. This enhanced matrix compute unit supports arbitrary matrix dimensions and 64-bit numerical precision. The design builds upon the original SME architecture while adding configurability for matrix sizes and increased data width

2. Architecture

2.1 Core Components Instruction Decoder : Parses assembly instructions and controls execution flow and matrix dimension information.

- Register File : Stores matrix operands with variable dimensions

- 64 Matrix ALU : Performs the actual matrix operations

- Control Unit : Manages execution for and multi-cycle operations

- Memory Interface : Handles loading/storing matrices from/to external memory

2.2 Matrix Dimensions and Format

- Support for matrices from 1x1 upto 32x32

- Element Precision: 64-bit fixed point or IEEE-754 double precision floating point

- Matrix storage: Row-major order with dimension meta data.

Section 3

3.1 Instruction Format

Each instruction is a 64-bit word with the following fields:

- Opcode    (6 bits): Operation to perform

- Destination Register    (4 bits): Target register (16 matrix registers)

- Source Register 1    (4 bits): First operand register

- Source Register 2    (4 bits): Second operand register

- Dimension Control    (10 bits): Matrix dimension information

  - Rows Source 1 (5 bits): Number of rows in first source matrix

  - Cols Source 1/Rows Source 2 (5 bits): Number of columns in first source/rows in second source

- Cols Source 2    (5 bits): Number of columns in second source matrix

- Format    (3 bits): Data format control (fixed-point, floating-point, etc.)

- Flags    (4 bits): Special operation modifiers

- Reserved    (24 bits): For future extensions

‘

3.2 Opcodes

- 000001  : MADD (Matrix Addition)

- 000010  : MSUB (Matrix Subtraction)

- 000011  : MMUL (Matrix Multiplication)

- 000100  : MDIV (Matrix Element-wise Division)

- 000101  : MLOAD (Load Matrix from Memory)

- 000110  : MSTORE (Store Matrix to Memory)

- 000111  : MTRANS (Matrix Transpose)

- 001000  : MZERO (Initialize Matrix with Zeros)

- 001001  : MIDENTITY (Initialize Matrix as Identity)

- 000110    : MINV (Matrix Inversion)

- 000111    : MDET (Matrix Determinant)

- 001000    : MEIG (Matrix Eigenvalues)

- 001001    : MSVD (Matrix Singular Value Decomposition)

- 001010    : MSIZE (Set Matrix Dimensions)

- 001011    : MSLICE (Extract Submatrix)

- 001100    : MCONCAT (Concatenate Matrices)

- 001101    : MTYPE (Change Matrix Element Type)

3.3 Assembly Syntax

[OPCODE] [DEST_REG], [SRC_REG_1], [SRC_REG_2], [DIM_ROWS], [DIM_COLS]

Example:

MADD M0, M1, M2

MSUB M3, M4, M5

MMUL M6, M7, M8

MDIV M9, M10, M11

MLOAD M0, M1, M2

MSTORE M1, M2, M3

MTRANS M4

MINV M5

MDET M6

MEIG M7

MSVD M8

MSIZE M9, M10

MSLICE M11, M12

MCONCAT M13, M14

MTYPE M15, M0

MZERO M0

MIDENTITY M1

MREDUCE M2, M3

MDISTRIBUTE M4, M5

MPARTITION M1, STRATEGY_ID

MBARRIER

4. Dynamic Memory Management

4.1 Matrix Descriptor Format

Each matrix register contains:

- Metadata Block:

  - Rows (5 bits): Number of rows (1-32)

  - Columns (5 bits): Number of columns (1-32)

  - Format (3 bits): Data format identifier

  - Memory Address (32 bits): Pointer to matrix data

- Data Block: Actual matrix elements

4.2 Memory Allocation Strategy

- On-chip SRAM for small matrices (≤ 8×8)

- External memory for larger matrices with DMA transfers

- Intelligent caching of frequently accessed elements

- Tiling approach for very large matrix operations

5. Execution Model for Variable-Sized Operations

5.1 Tiled Execution Engine

- Matrix operations broken down into tiles (8×8 typical size)

- Pipeline optimized for tile-based processing

- Multiple parallel execution units for tile operations

- Work-stealing scheduler for load balancing

5.2 Execution Time Estimation

- Addition/Subtraction: O(rows × cols) cycles

- Multiplication: O(rows × inner_dim × cols) cycles

- Division: O(rows × cols) cycles

- Inversion: O(dim³) cycles

5.1 External Interfaces

- Clock : System clock input

- Reset : Asynchronous system reset

- Instruction Input : 32-bit instruction word

- Data Memory Interface : For loading/storing matrices

- Control Signals :

- start  : Begin execution

- busy  : Indicates operation in progress

- done  : Indicates completion

- AXI4 Memory Interface: For high-bandwidth matrix transfers

- Configuration Registers: Control matrix engine parameters

- Status Registers: Report operation progress and error conditions

- Interrupt Controller: Signal operation completion or errors

5.2 Register Addresses

- Matrix Registers: M0-M7 (8 registers in total)

6. Error Handling

- Detection and reporting of singular matrices

- Numerical stability monitoring

- Dimension compatibility checking

- Overflow/underflow detection

7. Performance Goals

- Maximum clock frequency: 1Ghz

- Matrix multiplies throughput: 6.25 million elements per second

- Power consumption: Less than 100mW

Matrix Extension for Multi-Core Operation and Implementation

8. Multi-Core System Organization

8.1 Core Components

- Control Processor      : Orchestrates overall operation and task distribution

- Matrix Processing Elements (MPEs)      : 4-16 individual compute cores optimized for matrix operations

- Shared Memory Hierarchy      : Multi-level memory system with coherence protocol

- Interconnect Network      : High-bandwidth communications fabric connecting cores

- Global Scheduler      : Distributes matrix workloads across available cores

8.2 Matrix Processing Element Design

Each MPE contains:

- Local Register File      : 16 matrix registers with metadata

- 64-bit FPU      : Dedicated floating-point unit with multiply-accumulate capability

- Local Tiling Engine      : Manages processing of matrix subblocks

- L1 Matrix Cache      : High-bandwidth local storage (512KB per core)

- DMA Engine      : Autonomous data transfer with main memory

9. Workload Distribution

9.1 Matrix Partitioning Strategies

The system employs three main partitioning approaches:

- Block Division      : Splits matrices into equal-sized blocks assigned to different cores

- Row/Column Division      : Distributes complete rows or columns across cores

- Cyclic Assignment      : Interleaves matrix elements for load balancing

9.2 Automatic Partitioning Algorithm

- Analyze matrix dimensions and sparsity pattern

- Select optimal partitioning strategy based on operation type

- Create work packages with dependencies tracked

- Distribute packages to cores with load balancing

- Manage synchronization points

9.3 Dynamic Load Balancing

- Work-stealing queue implementation

- Runtime performance monitoring

- Adaptive core assignment based on thermal and power constraints

10 Inter-Core Communication

10.1 Communication Patterns

- Nearest-Neighbor      : For stencil operations on submatrices

- Broadcast      : Common operand distribution

- Reduction      : Gathering partial results

- All-to-All      : For matrix transpose and similar operations

10.2 Communication Infrastructure

- Crossbar switch for low-latency direct connections

- Multi-level NoC (Network-on-Chip) for scalability

- DMA-driven block transfers with prefetching

11. Memory Hierarchy

11.1 Memory Organization

- L1 Matrix Cache      : 512KB per core, highly banked for parallel access

- L2 Shared Cache      : 4MB shared among 4 cores

- L3 Global Cache      : 16-32MB with directory-based coherence

- Main Memory Interface      : Multiple channels to high-bandwidth memory

11.2 Coherence Protocol

- Directory-based MESI protocol

- Specialized for matrix block operations

- Optimized for producer-consumer relationships between cores

11.3 Memory Access Optimization

- Predictive prefetching based on access patterns

- Software-managed scratchpad for critical data

- Conflict-free banking for simultaneous access

12. Instruction Extension

12.1 Multi-Core Instructions

- MPARTITION     : Specify matrix partitioning strategy

- MDISTRIBUTE     : Distribute computation across cores

- MBARRIER     : Synchronize cores at critical points

- MREDUCE     : Combine partial results from multiple cores

12.2 Collective Operations

- Hardware-accelerated collective operations:

  - MMUL_COLLECTIVE     : Distributed matrix multiplication

  - MINV_COLLECTIVE     : Parallel matrix inversion

  - MSVD_COLLECTIVE     : Distributed singular value decomposition

13. Implementation Scalability

13.1 Core Scaling

- Base configuration: 4 cores

- Medium configuration: 8 cores

- High-performance configuration: 16 cores

- Enterprise configuration: 32+ cores with multi-chip scaling

13.2 Performance Scaling

- Matrix Addition: Near-linear scaling to 16 cores

- Matrix Multiplication: 85-95% efficiency scaling to 16 cores

- Matrix Inversion: 75-85% efficiency scaling to 16 cores

- SVD/Eigenvalues: 60-80% efficiency depending on matrix structure

Section 14

module multi_core_matrix_system (

input wire clk,               // System clock

input wire rst_n,             // Active low reset

input wire [63:0] instruction, // Instruction input

input wire start,             // Start execution signal

output wire busy,             // Execution in progress

output wire done,             // Execution complete

output wire error,            // Error indicator

// AXI4 Memory Interface

output wire [31:0] axi_araddr,

output wire axi_arvalid,

input wire axi_arready,

input wire [511:0] axi_rdata,

input wire [1:0] axi_rresp,

input wire axi_rlast,

input wire axi_rvalid,

output wire axi_rready,

output wire [31:0] axi_awaddr,

output wire axi_awvalid,

input wire axi_awready,

output wire [511:0] axi_wdata,

output wire [63:0] axi_wstrb,

output wire axi_wlast,

output wire axi_wvalid,

input wire axi_wready,

input wire [1:0] axi_bresp,

input wire axi_bvalid,

output wire axi_bready

);

// Number of matrix processing cores

parameter NUM_CORES = 4;

15. Test scenarios

- Execution trigger is all listed opcodes

- Flow for errors