SAMPLE AXI4-LITE PROTOCOL SPECIFICATION

Version 1.0

1. OVERVIEW

This document defines a simplified AXI4-Lite style protocol for communication between a master and a slave.

The protocol uses five independent channels:

1. Write Address Channel

2. Write Data Channel

3. Write Response Channel

4. Read Address Channel

5. Read Data Channel

All channels use a VALID/READY handshake mechanism.

2. CLOCK AND RESET

2.1 Clock

All protocol signals are synchronous to the rising edge of ACLK.

2.2 Reset

ARESETn is an active-low synchronous reset.

When ARESETn is LOW:

• All VALID signals must be LOW.

• Internal transaction state must return to IDLE.

• No transaction may be accepted.

• Write response state must be cleared.

• Read response state must be cleared.

3. WRITE ADDRESS CHANNEL

Signals:

AWADDR: 32-bit write address.

AWVALID: Master indicates that a valid write address is available.

AWREADY: Slave indicates that it can accept a write address.

Handshake:

A write address transfer occurs only when AWVALID == 1 AND AWREADY == 1.

The master must keep AWVALID HIGH until AWREADY becomes HIGH.

The slave must not accept an address when AWVALID is LOW.

The slave may deassert AWREADY when it cannot accept another address.

4. WRITE DATA CHANNEL

Signals:

WDATA: 32-bit write data.

WSTRB: 4-bit byte write enable.

WVALID: Master indicates valid write data.

WREADY: Slave indicates that it can accept write data.

Handshake:

A write data transfer occurs only when WVALID == 1 AND WREADY == 1.

The master must keep WVALID HIGH until WREADY becomes HIGH.

WSTRB[0] corresponds to WDATA[7:0].

WSTRB[1] corresponds to WDATA[15:8].

WSTRB[2] corresponds to WDATA[23:16].

WSTRB[3] corresponds to WDATA[31:24].

5. WRITE RESPONSE CHANNEL

Signals:

BRESP: 2-bit write response.

BVALID: Slave indicates that a response is available.

BREADY: Master indicates that it can accept the response.

BRESP values:

00 = OKAY

01 = SLVERR

A write response transfer occurs only when BVALID == 1 AND BREADY == 1.

BVALID must remain HIGH until BREADY becomes HIGH.

A successful write must generate an OKAY response.

6. READ ADDRESS CHANNEL

Signals:

ARADDR: 32-bit read address.

ARVALID: Master indicates valid read address.

ARREADY: Slave indicates that it can accept the address.

A read address transfer occurs only when ARVALID == 1 AND ARREADY == 1.

The master must keep ARVALID HIGH until ARREADY becomes HIGH.

7. READ DATA CHANNEL

Signals:

RDATA: 32-bit read data.

RRESP: 2-bit read response.

RVALID: Slave indicates that read data is available.

RREADY: Master indicates that it can accept read data.

A read data transfer occurs only when RVALID == 1 AND RREADY == 1.

RVALID must remain HIGH until RREADY becomes HIGH.

RRESP values:

00 = OKAY

01 = SLVERR

8. REGISTER ACCESS RULES

The slave contains the following registers:

CONTROL: Address 0x00000000

STATUS: Address 0x00000004

DATA: Address 0x00000008

CONTROL is read/write.

STATUS is read-only.

DATA is read/write.

Writing to STATUS must generate SLVERR.

Reading an undefined address must generate SLVERR.

Writing to an undefined address must generate SLVERR.

9. CONTROL REGISTER

CONTROL[0] = ENABLE

CONTROL[1] = START

CONTROL[7:2] are reserved.

Writing START = 1 starts the internal operation.

10. STATUS REGISTER

STATUS[0] = BUSY

STATUS[1] = DONE

STATUS[7:2] are reserved.

BUSY becomes HIGH when an operation starts.

BUSY becomes LOW when the operation completes.

DONE becomes HIGH for one clock cycle when the operation completes.

11. DATA REGISTER

DATA[31:0] stores a 32-bit data value.

A write to DATA updates its stored value.

A read from DATA returns its stored value.

12. TRANSACTION RULES

Only one write transaction may be active at a time.

Only one read transaction may be active at a time.

A new transaction must not overwrite an existing active transaction.

The slave must not lose accepted transactions.

13. TIMING RULES

All state transitions occur on the rising edge of ACLK.

VALID signals must not depend combinationally on READY signals.

READY may be combinational or registered.

Protocol outputs must remain stable while VALID is HIGH and READY is LOW.

14. ERROR HANDLING

Invalid register accesses generate SLVERR.

The slave must still complete the transaction after reporting SLVERR.

No transaction may remain permanently stuck because of an invalid address.